PyGeoParallax-IN

A 3D Ray-Tracing Parallax Correction Framework for

INSAT-3DR Geostationary Satellite Imagery over Complex Terrain

Project Reference Document

4-Month Implementation & Validation Plan

Domain: South Asia (INSAT-3DR field of view)

Focus Regions: Himalayas | Western Ghats | Central India

# 1. Overview & Motivation

Geostationary sensors such as INSAT-3DR view the Earth at oblique angles away from nadir. When a sensor observes an elevated cloud top — a deep convective tower over the Himalayas (12–16 km altitude) or an orographic cloud layer along the Western Ghats (3–6 km altitude) — the apparent ground position of that cloud is displaced several kilometers from its true location directly beneath the cloud. This effect is known as geometric parallax.

For INSAT-3DR, stationed at 74°E over the equator, viewing zenith angles over northern India (30°–35°N) and the Western Ghats (10°–20°N) produce parallax displacements of roughly 4 to 12 km, equivalent to 1 to 3 INSAT pixels. When such uncorrected imagery is used to train deep learning models against nadir-viewing references — GPM DPR spaceborne radar or IMD ground radar — the mismatch introduces systematic label misalignment and degrades performance at cloud edges.

PyGeoParallax-IN is a 4-month project to build an analytical 3D ray-tracing correction model that combines INSAT Cloud-Top Height (CTH) retrievals, ERA5 thermal profiles, and SRTM terrain elevation to re-project INSAT-3DR observations onto their true WGS84 ground position — producing a geometrically corrected imagery product suitable for multi-sensor training and validation.

# 2. Key Datasets & Specifications

Six datasets anchor the pipeline: INSAT-3DR radiometric and cloud-top height products, ERA5 reanalysis, GPM DPR and IMD radar as reference truth, and SRTM elevation for terrain correction.

| Dataset | Product / Source | Native Resolution | Coverage | Purpose |
| --- | --- | --- | --- | --- |
| INSAT-3DR Imager L1C | ISRO MOSDAC (HDF5) | 4 km (TIR1, TIR2, WV, MIR) | South Asia (5–37°N, 68–98°E), 30-min repeat | Primary radiometric channels to be geometrically corrected |
| INSAT-3DR Sounder / Imager CTH | ISRO MOSDAC L2 | 4 km × 4 km / 10 km × 10 km | Domain-wide, 30-min / 1-hour | Primary Cloud-Top Height (H_CTH) estimate |
| ERA5 Reanalysis | Copernicus CDS | 0.25° × 0.25°, 37 pressure levels | Hourly, global | Fallback CTH via T_B–T(z) profile matching |
| GPM DPR Level-2 | 2ADPR v07 (NASA PPS) | 5 km swath | Overpasses (JJAS 2022–2024) | Nadir reference labels (precipitation type & reflectivity) |
| IMD Doppler Weather Radar | IMD Archive / MOSDAC | 1 km × 1 km, 3D volume scans | Western Ghats (Mumbai, Goa, Kochi) & Himalayas (Agartala) | Independent ground-truth validation of cloud edge/core alignment |
| SRTM DEM | NASA JPL / USGS | 90 m (aggregated to 4 km) | South Asia domain | Topographic surface elevation (H_terrain) baseline |

# 3. Mathematical & Algorithmic Methodology

The correction proceeds in five stages: (1) locate the satellite in Earth-centered coordinates, (2) build the line-of-sight ray to each apparent pixel, (3) intersect that ray with the true cloud-top surface, (4) project down to the terrain to recover the true ground position, and (5) resample the warped grid back onto a regular lattice.

## Step 1.  Geocentric ECEF Representation

The Earth is modeled as a WGS84 reference ellipsoid with semi-major axis a = 6378.137 km and semi-minor axis b = 6356.7523 km. The INSAT-3DR satellite position in Earth-Centered Earth-Fixed (ECEF) coordinates is:

S⃗ = [ RS cos(φS) cos(λS) ; RS cos(φS) sin(λS) ; RS sin(φS) ]

where λₛ = 74.0°E, φₛ = 0.0°, and Rₛ = a + hₛ ≈ 42164.14 km.

## Step 2.  Apparent Line-of-Sight Vector

For an apparent INSAT pixel with geodetic coordinates (φₐₚₚ, λₐₚₚ) on the ellipsoid, its ECEF surface vector P⃗ₐₚₚ = [x₀, y₀, z₀]ᵀ is computed. The unit line-of-sight vector û from the satellite through the apparent ground pixel is:

û = ( P⃗app − S⃗ )  /  ‖ P⃗app − S⃗ ‖

## Step 3.  Cloud-Top 3D Intersection

The actual cloud top C is located at physical height H_CTH above the ellipsoid, along the line S⃗ + t·û. The parameter t is found by solving the expanded ellipsoid equation:

(xS + t·ux)² / (a + HCTH)²   +   (yS + t·uy)² / (a + HCTH)²   +   (zS + t·uz)² / (b + HCTH)²   =   1

Solving this quadratic for the smallest positive root t_cloud gives the 3D ECEF coordinates of the cloud top, C⃗ = [xᴄ, yᴄ, zᴄ]ᵀ.

## Step 4.  Topographic Terrain Correction & Nadir Projection

The true ground position (φ_true, λ_true) is found by projecting C⃗ orthogonally down to the terrain surface elevation H_terrain(φ, λ), taken from the SRTM DEM. The physical parallax displacement is:

Δd⃗ = GreatCircleDistance( (φapp, λapp),  (φtrue, λtrue) )

## Step 5.  Forward Resampling & Gap Filling

Parallax correction warps the pixel grid: high-altitude clouds shift outward, low-altitude clouds shift only slightly. To avoid grid holes and overlapping destination cells, a forward BucketResampler is used:

- When multiple shifted pixels land in the same destination cell, the pixel with the highest H_CTH is kept — the higher cloud top physically occludes what is beneath it.

- Empty cells created by cloud-top dilation are filled using anisotropic inverse-distance weighting (IDW), constrained along the local atmospheric wind vector.

# 4. Detailed 4-Month Implementation Roadmap

## Month 1 — Pipeline Setup, Data Engineering & Spatial Transformations

Week 1  Data Acquisition & Standardized Ingestion

- Download 3 monsoon seasons (JJAS 2022–2024) of INSAT-3DR Imager L1C, INSAT-3DR Sounder CTH, GPM DPR 2ADPR, and SRTM DEM data over South Asia.

- Build ingest parsers in Python (h5py, netCDF4, pyproj) to standardize raw instrument scans onto a uniform 0.04° × 0.04° grid.

Week 2  Viewing Geometry & Angle Calculation

- Compute pixel-level Satellite Viewing Zenith Angle (VZA), Viewing Azimuth Angle (VAA), and Solar Zenith Angle (SZA) for every INSAT-3DR scene from orbital state vectors.

- Map VZA distribution across India: 15°–25° over the Western Ghats, 30°–40° over the Himalayas.

Week 3  Geocentric ECEF Transformation Engine

- Implement forward and inverse transforms between geodetic (φ, λ, h) and WGS84 ECEF [x, y, z] coordinates.

- Unit-test against pyproj benchmark coordinates, targeting sub-meter precision.

Week 4  Cloud-Top Height (CTH) Ingestion & Fallback Module

- Integrate INSAT-3DR Sounder CTH products.

- Build a thermal-profile lookup using hourly ERA5 T(z): where satellite CTH is missing, match TIR1 brightness temperature to the ERA5 profile to estimate H_CTH = z(T_B).

## Month 2 — Ray-Tracing Engine, Terrain Integration & Resampling

Week 5  Analytical Ellipsoid Ray-Tracing Solver

- Implement the quadratic ellipsoid solver for t_cloud and the 3D cloud-top ECEF point C⃗.

- Vectorize with NumPy and Numba/PyCUDA to process a full 1000 × 1000 INSAT disk frame in under 2 seconds.

Week 6  SRTM Topographic Relief Integration

- Bring high-resolution SRTM elevation into the projection loop to capture slope geometry along the Western Ghats escarpment and Himalayan ridges.

- Derive shift maps of theoretical parallax displacement (km) as a function of CTH (2–16 km) and location.

Week 7  BucketResampler & Gridding Strategy

- Build the BucketResampler for destination-grid allocation.

- Handle overlap (max-CTH wins for occluding clouds) and sparsity (terrain-aware local interpolation for single-pixel gaps).

Week 8  Pipeline Automation & Batch Processing

- Connect all stages into one pipeline: INSAT L1C + CTH + DEM → Ray-Tracing → Resampling → corrected GeoTIFF/NetCDF.

- Batch-test on 100 benchmark monsoon storm scenes from JJAS 2023.

## Month 3 — Multi-Sensor Validation & Quantitative Alignment Analysis

Week 9  Spaceborne Radar (GPM DPR) Cross-Alignment

- Colocate uncorrected and corrected INSAT-3DR scenes with overlapping GPM DPR orbits (±10 min window).

- Compute spatial cross-correlation and mutual information between INSAT TIR1 cloud edges and GPM DPR Ku-band 18 dBZ echo-top boundaries.

Week 10  Ground Radar (IMD DWR) High-Resolution Validation

- Extract 3D volume scans from IMD DWR stations at Mumbai, Goa, Kochi (Western Ghats) and Agartala (Northeast India).

- Compute Intersection-over-Union between INSAT deep-convective masks (T_B < 220 K) and DWR 35 dBZ reflectivity cores, before vs. after correction.

Week 11  Regional Statistical Dissection

- Partition results by regime: Western Ghats (orographic shallow rain vs. deep cores), Himalayan Foothills (high VZA + high relief), Central India Plains (flat-terrain control).

- Verify shift vectors: Himalayan convective cores shift south-southwest by 6–10 km; Western Ghats cores shift southwest by 3–6 km.

Week 12  Sensitivity & Error Budget Analysis

- Run Monte Carlo simulations perturbing CTH by ±1 km and ±2 km to quantify how CTH retrieval error propagates into horizontal geolocation error.

- Establish confidence bounds for the correction framework.

## Month 4 — Downstream Model Evaluation, Manuscript & Code Release

Week 13  Impact Assessment on Deep Learning Segmentation

- Train two identical 8-channel U-Net precipitation classification models: Model A on uncorrected INSAT imagery + GPM DPR labels; Model B on parallax-corrected imagery + GPM DPR labels.

- Evaluate over the Western Ghats and Himalayan test domains to verify reduced edge misclassification and false negatives.

Week 14  Manuscript Structuring & Figure Generation

- Draft: Introduction, Physical Geometry, Methodology, Multi-Sensor Validation Results, Deep Learning Impact, Discussion.

- Generate figures: 3D ray-tracing schematics, parallax displacement vector maps over India, DWR–satellite overlays, IoU/cross-correlation charts.

Week 15  Manuscript Writing & Internal Peer Review

- Finalize text, write LaTeX source, format mathematical proofs, conduct self-review.

Week 16  Code Packaging & Submission

- Clean, document, and publish the open-source PyGeoParallax-IN Python package on GitHub with tutorial Jupyter notebooks.

- Submit manuscript to target journal.

# 5. Expected Outcomes & Deliverables

## 5.1  Quantifiable Performance Targets

| Evaluation Metric | Baseline (Uncorrected) | Target (Corrected) | Improvement |
| --- | --- | --- | --- |
| GPM DPR Echo-Top Boundary IoU | 0.42–0.48 | 0.68–0.75 | +40–50% spatial overlap |
| IMD DWR Core Cross-Correlation | 0.51 | 0.82 | +0.31 correlation gain |
| Mean Geolocation Shift Distance | 0.0 km (uncorrected error) | 4.2 km (Ghats) / 8.6 km (Himalayas) | Physical alignment restored |
| Downstream U-Net Edge Precision | 0.45 | 0.62 | +17% boundary precision |

## 5.2  Open-Source Software Package

PyGeoParallax-IN: a modular Python library, accelerated with PyCUDA/Numba, for rapid 3D ray-tracing parallax correction of geostationary imagery (INSAT-3DR, INSAT-3DS) over complex topography.